Sound Localization Test — Implementation Task Checklist

Version 1.0  ·  2026-04-13  ·  All tasks must be completed in order.

Section 1 — Project Setup

☐  1.1 Create the project folder structure: Sound Localization Test/, documents/, archive/, results/

☐  1.2 Create SLT.m with a file header: title, description, author, date, version.

☐  1.3 Define global configuration constants at the top: sample rate, speaker angles (0°–300°), Hann ramp duration, number of channels (6).

Section 2 — Audio Engine

☐  2.1 Write generateStimulus(frequency, sampleRate, duration) — generates one buffer of the selected stimulus: Hann-windowed sine wave or Gaussian noise; returns a mono audio vector.

☐  2.2 Write buildOutputBuffer(monoSignal, channelAmplitudes, numChannels) — maps a mono signal to a 6-channel output matrix by scaling each channel by its assigned amplitude.

☐  2.3 Write computePanAmplitudes(degree, speakerAngles) — implements sine law panning; takes a degree (1–360), identifies the bounding speaker pair, returns a 6-element amplitude vector.

☐  2.4 Write initAudio(sampleRate, numChannels) — initializes the multi-channel audio device via PsychPortAudio; returns a device handle.

☐  2.5 Write playLooping(deviceHandle, buffer) — begins looped, continuous playback of the buffer on the audio device.

☐  2.6 Write stopAudio(deviceHandle) — stops playback and releases the audio device cleanly.

Section 3 — Intro GUI

☐  3.1 Build the Intro GUI figure window with correct background color and layout proportions.

☐  3.2 Add teal header banner with application title.

☐  3.3 Add Stimulus Selection dropdown (125 Hz, 250 Hz, 500 Hz, 750 Hz, 1000 Hz, Gaussian Noise).

☐  3.4 Add Number of Trials numeric input field.

☐  3.5 Add Pause Time (s) numeric input field.

☐  3.6 Add Mode Selection dropdown (Discrete Speakers, Continuous Panning).

☐  3.7 Add Description text input field.

☐  3.8 Add green Start button with callback: reads all field values, validates inputs, launches Subject ID prompt.

☐  3.9 Add Calibrate button with callback: reads selected stimulus, launches calibration routine.

☐  3.10 Write input validation: Number of Trials must be a positive integer; Pause Time must be a positive number; warning dialog on failure.

Section 4 — Subject ID Prompt

☐  4.1 Write getSubjectID() — modal input dialog prompting for Subject ID; returns the entered string.

☐  4.2 Construct session filename from Subject ID and current timestamp (SubjectID_YYYYMMDD_HHMMSS).

Section 5 — Calibration Routine

☐  5.1 Build the Calibration GUI: displays current speaker number and channel, Next Speaker button.

☐  5.2 Write the calibration loop: for speakers 1–6, play stimulus on corresponding channel, update display, wait for Next Speaker button.

☐  5.3 Stop audio and close Calibration GUI after Speaker 6.

Section 6 — Experiment Engine

☐  6.1 Write selectStimulusLocation(mode) — returns a random speaker (1–6) for Discrete or a random integer degree (1–360) for Continuous.

☐  6.2 Write the main experiment loop: draw experiment screen → select location → build 6-channel buffer → start looped playback → start timer → wait for response → stop timer/audio → record data → display countdown. Repeat for N trials.

☐  6.3 Implement Discrete response capture: listen for keypresses 1–6; record and convert to speaker number.

☐  6.4 Implement Continuous response capture: render circular 360° response ring; record click angle.

☐  6.5 Write countdown pause display: show remaining seconds updating in real time between trials.

☐  6.6 Compute angular error per trial: absolute angular difference between stimulus and response accounting for circular wraparound.

Section 7 — Results Computation & Display

☐  7.1 Compute summary statistics: total accuracy (Discrete), mean angular error, mean response time.

☐  7.2 Build Heatmap 1 (Angular Error): x-axis = stimulus locations, color = mean error; red=high, green=low; title and Description subheader.

☐  7.3 Build Heatmap 2 (Response Time): x-axis = stimulus locations, color = mean response time; red=high, green=low; title and Description subheader.

☐  7.4 Display Results GUI/figure showing both heatmaps and summary statistics text.

☐  7.5 Save both heatmaps as .png files to results/ using the session filename.

☐  7.6 Write trial data to CSV in results/ with columns: Trial Number, Stimulus Location, Response, Angular Error, Response Time.

Section 8 — Integration & Testing

☐  8.1 Connect Intro GUI → Subject ID prompt → Experiment loop → Results in a single clean execution flow.

☐  8.2 Connect Calibrate button → Calibration routine → return to Intro GUI.

☐  8.3 Test Discrete Speakers mode end-to-end with a small number of trials.

☐  8.4 Test Continuous Panning mode end-to-end with a small number of trials.

☐  8.5 Verify sine law panning: at exact speaker angles only one channel outputs; at mid-sector power is split equally.

☐  8.6 Verify CSV output format and file naming.

☐  8.7 Verify figure output: both heatmaps saved with proper titles and Description subheaders.

☐  8.8 Verify audio cleanup: no hanging audio processes after experiment completes or is interrupted.

Document status: Awaiting user approval before implementation begins.