Sound Localization Test — Product Requirement Document

Version 1.1  ·  2026-04-13  ·  Author: Claude (Anthropic) in collaboration with project owner

# 1. Project Overview

The Sound Localization Test (SLT) is a MATLAB-based psychoacoustic experiment that assesses a human listener's ability to localize sound sources in the horizontal plane. The program plays a stimulus from one of six spatially arranged speakers — or from a virtual panned position between two speakers — and records where the listener perceives the sound to originate. Results are analyzed and visualized automatically at the end of each session.

# 2. Physical System

Six loudspeakers are arranged in a regular hexagonal formation centered on the listener. Speakers are numbered 1–6 clockwise, with Speaker 1 directly in front (0°). Remaining speakers are at 60° increments:

| Speaker | Angle | Position |
| --- | --- | --- |
| 1 | 0° | Front |
| 2 | 60° | Front-Right |
| 3 | 120° | Back-Right |
| 4 | 180° | Back |
| 5 | 240° | Back-Left |
| 6 | 300° | Front-Left |

Audio is routed via a multi-channel audio interface using MATLAB's Audio Toolbox (audioPlayerRecorder). MATLAB audio channels 1–6 map directly to speakers 1–6. Psychtoolbox is not required; the Audio Toolbox provides sufficient timing precision for a reaction-time localization experiment where response latency is on the order of hundreds of milliseconds to seconds.

# 3. Acoustic & Signal Processing Context

## 3.1 Stimulus Types

- Tonal stimuli: Hann-windowed sine waves at 125, 250, 500, 750, or 1000 Hz. The window eliminates spectral splatter and audible clicks at onset/offset. The windowed buffer is looped continuously until the listener responds.

- Gaussian noise: White Gaussian noise generated fresh each trial, looped continuously until the listener responds.

## 3.2 Discrete Speaker Mode

A single speaker (1–6) is chosen at random each trial (fully random, repeats allowed). The stimulus plays from that channel at full amplitude on the corresponding audio channel.

## 3.3 Continuous Panning Mode — Sine Law Panning

A random integer degree (1–360) is selected each trial. The 360° space is divided into six 60° sectors, each bounded by two adjacent speakers:

| Sector | Bounding Speakers | Degree Range |
| --- | --- | --- |
| 1 | Speakers 1 & 2 | 0° – 60° |
| 2 | Speakers 2 & 3 | 60° – 120° |
| 3 | Speakers 3 & 4 | 120° – 180° |
| 4 | Speakers 4 & 5 | 180° – 240° |
| 5 | Speakers 5 & 6 | 240° – 300° |
| 6 | Speakers 6 & 1 | 300° – 360° |

Within a sector, amplitude is split between the two bounding speakers using the sine panning law to maintain constant total power:

theta = (degree - sector_start) / 60 * (pi/2)

A_left_speaker  = cos(theta)

A_right_speaker = sin(theta)

When theta = 0, sound plays entirely from the counterclockwise speaker. When theta = pi/2, it plays entirely from the clockwise speaker. Intermediate values distribute power continuously between the pair.

# 4. GUI Design

The GUI aesthetic follows a clean laboratory style: teal/cyan header banners, soft pink input fields, white panel backgrounds, light blue overall background, and a bold green Start button.

## 4.1 Intro GUI

| Element | Type | Details |
| --- | --- | --- |
| Stimulus Selection | Dropdown | 125 Hz, 250 Hz, 500 Hz, 750 Hz, 1000 Hz, Gaussian Noise |
| Number of Trials | Numeric input | Positive integer |
| Pause Time | Numeric input | Seconds (positive number) |
| Mode Selection | Dropdown | Discrete Speakers / Continuous Panning |
| Description | Text field | Free text — used as figure subheader in results |
| Start | Button | Launches Subject ID prompt then experiment |
| Calibrate | Button | Launches calibration routine |

## 4.2 Calibration Routine

On pressing Calibrate, the program cycles through speakers 1–6. For each speaker it displays the active channel, plays the selected stimulus continuously, and waits for the user to click Next Speaker before advancing. Audio stops and the window closes after Speaker 6.

## 4.3 Experiment Screen — Discrete Speakers Mode

- Neutral/black background — no speaker diagram or visual cues.

- Displays current trial number and total trials (e.g., Trial 3 of 20).

- Listens for keypress 1–6; stops stimulus immediately on keypress.

- Displays a live countdown timer for the configured pause duration before the next trial.

## 4.4 Experiment Screen — Continuous Panning Mode

- Neutral/black background.

- Displays current trial number.

- Displays an interactive circular ring (0–360°) the listener clicks to indicate perceived direction.

- Stops stimulus immediately on click; records the clicked degree as the response.

- Displays live countdown timer before next trial.

## 4.5 Subject ID Prompt

A modal dialog prompts for Subject ID (text entry) on pressing Start. The session filename is constructed from Subject ID and a timestamp (SubjectID_YYYYMMDD_HHMMSS).

# 5. Experiment Logic

- Subject ID is collected.

- For each trial: location selected at random → stimulus plays → response timer starts → listener responds → timer stops → stimulus stops → data recorded → countdown pause displays.

- After the final trial, results are computed and displayed.

# 6. Results

## 6.1 Summary Statistics

- Total accuracy (Discrete mode only): percentage of trials where the correct speaker was selected.

- Average angular error: mean absolute angular difference between stimulus and response in degrees, accounting for circular wraparound.

- Average response time: mean time in seconds from stimulus onset to listener response.

## 6.2 Heatmap Visualizations

Two circular heatmap figures are generated per session, one for angular error and one for response time. Both share the same design specification below. Color encodes the metric: red = high, green = low, with yellow in between.

Discrete Speakers mode: the circle is divided into 6 solid wedges of 60° each. Each wedge is centered on its speaker angle (0°, 60°, 120°, 180°, 240°, 300°) so that the speaker angle bisects the wedge. Hard boundaries between wedges. The measured value for each speaker is printed inside its wedge.

Continuous Panning mode: the circle is divided into 36 wedge bins of 10° each, running from center to rim. Each bin is centered on its degree (0°, 10°, 20°, ... 350°) and colored by the average metric value for trials whose stimulus fell in that bin. The 6 speaker positions are labeled around the outside (e.g. '1 · 0°') along with intermediate tick marks and degree labels at 30° intervals to provide spatial reference without crowding.

Both modes: a top-down listener head icon (circle with eyes, ears, and a forward-pointing nose) is rendered in the center of the heatmap, oriented so the nose points toward Speaker 1 (0°, front). This provides an unambiguous orientation reference. Speaker labels appear at radius 158 px from center. The Description field from the Intro GUI is used as a subheader on both figures. Titles reflect their content clearly.

## 6.3 Data Export

- CSV file: one row per trial. Columns: Trial Number, Stimulus Location, Response, Angular Error (degrees), Response Time (seconds).

- Both heatmap figures saved as .png files.

- All files saved to results/ using the session filename.

# 7. Code Standards

- Highly readable: clear variable names, logical structure, generous section headers and inline comments.

- Physics/acoustics-forward: comments explain the perceptual and acoustic meaning of equations and parameters.

- Efficient: vectorized operations where appropriate; no redundant computation.

- Accessible: a reader with minimal software experience should be able to follow the code.

Document status: Awaiting user approval before implementation begins.