Sound Localization Test — Implementation Task Checklist

Version 1.2  ·  2026-04-13  ·  Complete all tasks in order.

Section 1 — Project Setup

☐  1.1 Create the project folder structure: Sound Localization Test/, documents/, archive/, results/

☐  1.2 Create SLT.m with a file header: title, description, author, date, version.

☐  1.3 Define global configuration constants: sample rate (48000 Hz), speaker angles (0°/60°/120°/180°/240°/300°), Hann ramp duration (10 ms), crossfade duration (10 ms), number of channels (6).

Section 2 — Audio Engine

☐  2.1 Write buildToneBuffer(frequency, sampleRate) — integer-cycle sine wave loop buffer (N = Fs/f samples). No windowing applied to the buffer itself.

☐  2.2 Write buildNoiseBuffer(sampleRate, durationSeconds) — Gaussian noise buffer with 10 ms equal-power crossfade stitched at the loop boundary.

☐  2.3 Write applyOnsetRamp(buffer, sampleRate, rampMs) — one-sided half-Hann fade-in applied to the first rampMs ms of a buffer; used once per trial at stimulus onset.

☐  2.4 Write applyOffsetRamp(buffer, sampleRate, rampMs) — one-sided half-Hann fade-out applied to the last rampMs ms; used once per trial at stimulus offset.

☐  2.5 Write buildOutputBuffer(monoSignal, channelAmplitudes, numChannels) — maps mono signal to 6-channel output matrix.

☐  2.6 Write computePanAmplitudes(degree, speakerAngles) — sine law panning; returns 6-element amplitude vector.

☐  2.7 Write initAudio(sampleRate, numChannels) — initializes multi-channel device via audioPlayerRecorder; returns device handle.

☐  2.8 Write playLooping(deviceHandle, buffer) — begins looped continuous playback.

☐  2.9 Write stopAudio(deviceHandle) — stops playback and releases device cleanly.

Section 3 — Intro GUI

☐  3.1 Build Intro GUI figure window with correct background color and layout.

☐  3.2 Add teal header banner with application title.

☐  3.3 Add Stimulus Selection dropdown (125 Hz, 250 Hz, 500 Hz, 750 Hz, 1000 Hz, Gaussian Noise).

☐  3.4 Add Number of Trials numeric input field.

☐  3.5 Add Pause Time (s) numeric input field.

☐  3.6 Add Mode Selection dropdown (Discrete Speakers, Continuous Panning).

☐  3.7 Add Description text input field.

☐  3.8 Add green Start button: reads all fields, validates inputs, launches Subject ID prompt.

☐  3.9 Add Calibrate button: reads selected stimulus, launches calibration routine.

☐  3.10 Write input validation: Number of Trials must be positive integer; Pause Time must be positive number; warning dialog on failure.

Section 4 — Subject ID Prompt

☐  4.1 Write getSubjectID() — modal input dialog; returns entered Subject ID string.

☐  4.2 Construct session filename: SubjectID_YYYYMMDD_HHMMSS.

Section 5 — Calibration Routine

☐  5.1 Build Calibration GUI: displays current speaker number and channel, Next Speaker button.

☐  5.2 Write calibration loop: build appropriate loop buffer → play on corresponding channel → update display → wait for Next Speaker click.

☐  5.3 Stop audio and close Calibration GUI after Speaker 6.

Section 6 — Experiment Engine

☐  6.1 Write selectStimulusLocation(mode) — random speaker (1–6) for Discrete; random integer degree (1–360) for Continuous.

☐  6.2 Write main experiment loop: draw screen → select location → build loop buffer → apply onset ramp → build 6-channel buffer → start looped playback → start timer → wait for response → apply offset ramp → stop audio → record data → display countdown. Repeat N trials.

☐  6.3 Implement Discrete response capture: listen for keypresses 1–6; record and convert to speaker number.

☐  6.4 Implement Continuous response capture: render circular 360° response ring; record click angle.

☐  6.5 Write countdown pause display: live seconds countdown between trials.

☐  6.6 Compute angular error per trial: absolute angular difference accounting for circular wraparound.

Section 7 — Results Computation & Display

☐  7.1 Compute summary statistics: total accuracy (Discrete), mean angular error, mean response time.

☐  7.2 Write drawCircularHeatmap(values, mode, metric, description) — per PRD spec: 6 centered solid wedges (Discrete) or 36×10° wedges (Continuous); listener head at center; speaker labels and degree ticks; green/yellow/red scale; description subheader.

☐  7.3 Call drawCircularHeatmap twice: once for angular error, once for response time.

☐  7.4 Display Results figure showing both heatmaps and summary statistics text.

☐  7.5 Save both heatmaps as .png to results/ using session filename.

☐  7.6 Write trial data to CSV in results/: Trial Number, Stimulus Location, Response, Angular Error, Response Time.

Section 8 — Integration & Testing

☐  8.1 Connect Intro GUI → Subject ID prompt → Experiment loop → Results in single clean execution flow.

☐  8.2 Connect Calibrate button → Calibration routine → return to Intro GUI.

☐  8.3 Test Discrete Speakers mode end-to-end.

☐  8.4 Test Continuous Panning mode end-to-end.

☐  8.5 Verify tone loop buffer: no audible click or modulation during looping; confirm integer-cycle length for all 5 frequencies at 48000 Hz.

☐  8.6 Verify noise loop buffer: no audible click at loop boundary; crossfade maintains constant power.

☐  8.7 Verify Hann ramps: onset and offset ramps applied exactly once per trial, not on every loop cycle.

☐  8.8 Verify sine law panning: at exact speaker angles only one channel outputs; at mid-sector power is equal.

☐  8.9 Verify heatmap geometry: all wedges fill the full circle; listener head centered and oriented correctly.

☐  8.10 Verify CSV output format and file naming.

☐  8.11 Verify figure output: both heatmaps saved with correct titles and description subheaders.

☐  8.12 Verify audio cleanup: no hanging audio processes after experiment completes or is interrupted.

Document status: Awaiting user approval before implementation begins.